## 1. Device & Hardware Configuration

| Component | Configuration |
| --- | --- |
| MCU | ESP32-S3 @ 240 MHz |
| PCB | C6 PCB |
| Transport | BLE (WRITE_NR) + Streaming Ring Buffer |
| PSRAM | 8192 KB |
| Free PSRAM (idle) | ~8189 KB |
| Camera | OV3660 |
| PTT Button | GPIO5 |

## 2. GPIO & Interface Mapping

| Function | Pins / Interface |
| --- | --- |
| Speaker (I2S) | BCLK=GPIO6, LRC=GPIO7, DIN=GPIO8 |
| Microphone | Built-in PDM (CLK=GPIO42, DATA=GPIO41) |
| Camera XCLK | GPIO10 |
| PTT Button | GPIO5 |

## 3. Audio Configuration

### Microphone

| Parameter | Value |
| --- | --- |
| Type | Built-in PDM (MSM261D3526H1CPM) |
| Sample Rate | 16,000 Hz |
| Bit Depth | 16-bit |
| Channels | Mono |
| DMA Chunk Size | 512 samples (32 ms) |

### Speaker

| Parameter | Value |
| --- | --- |
| Interface | I2S STD |
| Sample Rate | 22,050 Hz |
| Bit Depth | 16-bit |
| Channels | Stereo (L=R, dual-amp) |
| Codec / Amp | MAX98357A |

## 4. Buffering & Streaming Parameters

| Item | Value |
| --- | --- |
| Ring Buffer Size | 160 KB |
| Max Audio Stored | ~3.72 s @ 22050 Hz mono |
| Pre-buffer Threshold | 24,000 bytes (~544 ms) |
| Streaming Underruns | 0% |

## 5. BLE Configuration & Throughput

| Parameter | Value |
| --- | --- |
| MTU | 512 bytes |
| Audio Payload / Packet | 507 bytes |
| Packets per Audio Reply | 256 |
| BLE RX Throughput (ESP32) | ~39 KB/s |
| BLE TX Throughput (Android – Voice) | ~48.7 KB/s |
| BLE TX Throughput (Android – Vision) | ~50 KB/s |

## 6. Memory Usage (ESP32-S3)

| Stage | Free Heap | Free PSRAM |
| --- | --- | --- |
| After Boot | ~297 KB | ~8189 KB |
| After Mic Init | ~271 KB | ~7994 KB |
| After BLE Init | ~208 KB | ~7994 KB |
| After Playback | ~189 KB | ~7994 KB |
| Min Ever Heap | ~190 KB | — |

## 7. Voice-Only Pipeline Latency Breakdown

| Stage | Duration |
| --- | --- |
| Whisper (STT) | 8,582 ms |
| GPT Response | 2,417 ms |
| TTS API | 2,954 ms |
| MP3 Decode | 694 ms |
| Pipeline Total | 14,663 ms (14.7 s) |
| BLE TX (Android → Glasses) | 2,591 ms |

## 8. Vision Pipeline Latency Breakdown (Photo + Voice)

| Stage | Duration |
| --- | --- |
| Whisper (Vision) | 1,224 ms |
| Vision API | 2,453 ms |
| TTS API (Vision) | 4,352 ms |
| MP3 Decode (Vision) | 3,553 ms |
| Pipeline Total (Vision) | 11,599 ms (11.6 s) |
| BLE TX (Vision) | 13,551 ms |

## 9. Image Transfer Performance (Vision Mode)

| Metric | Value |
| --- | --- |
| Image Size | ~7.6 KB |
| Image RX Time | 282 ms |
| Image Throughput | ~27 KB/s |

## 10. End-to-End Audio Playback Metrics (ESP32)

| Metric | Value |
| --- | --- |
| Playback Start Delay | ~551 ms |
| Total Audio Received | 129,124 bytes |
| Dropped Packets | 0 |
| Lost Packets | 0 |
| Round-Trip (PTT release → playback end) | ~18.18 s |

### Want next steps?

| Metric | Derived |
| --- | --- |
| Whisper (voice) | 16:02:01.072 − 16:01:52.490 = 8,582 ms |
| GPT | 16:02:03.489 − 16:02:01.072 = 2,417 ms |
| TTS API | 16:02:06.443 − 16:02:03.489 = 2,954 ms |
| MP3 decode | 16:02:07.149 − 16:02:06.455 = 694 ms |
| Pipeline total | 16:02:07.149 − 16:01:52.486 = 14,663 ms |
| BLE TX (Android) | 16:02:09.743 − 16:02:07.152 = 2,591 ms → 48.7 KB/s |
| Whisper (vision) | 16:05:32.533 − 16:05:31.311 = 1,224 ms |
| Vision API | 16:05:34.986 − 16:05:32.533 = 2,453 ms |
| TTS API (vision) | 16:05:39.339 − 16:05:34.987 = 4,352 ms |
| MP3 decode (vision) | 16:05:42.903 − 16:05:39.350 = 3,553 ms |
| Pipeline total (vision) | 16:05:42.908 − 16:05:31.309 = 11,599 ms |
| BLE TX (vision) | 16:05:56.459 − 16:05:42.908 = 13,551 ms → 50 KB/s |
| Image RX (Android) | 16:05:26.176 − 16:05:25.894 = 282 ms → 27 KB/s |