S3_App_imp

Performance Metrics Report

ESP32-S3 BLE Voice + Vision Assistant — Full-Stack Instrumentation

Firmware (Arduino / NimBLE / IDF I2S v2)   +   Android (Kotlin / BLE GATT)

Branch: perf-instrumentation-s3  |  Date: 2026-04-13

# 1. Overview

This report documents full-stack performance instrumentation applied to the S3_App_imp project (ESP32-S3 BLE Voice + Vision Assistant). Timing and performance log statements were inserted into the three source files listed below. The code changes live exclusively on the perf-instrumentation-s3 branch and have not been merged to main.

Instrumented files:

- S3_App_imp/S3_App_imp.ino — ESP32-S3 Arduino firmware (NimBLE + IDF I2S v2 + Camera)

- AIglasses/…/BleVoiceService.kt — Android BLE GATT client

- AIglasses/…/VoiceAssistantPipeline.kt — Android processing pipeline

All 19 metrics are covered. Static values are derived from source code analysis. Runtime values are emitted by the instrumented log statements — see the log-tag column in each section.

Key S3 architectural differences vs C6 (affects several metrics):

- Independent I2S ports: PDM mic on I2S_NUM_0, speaker STD on I2S_NUM_1 — NO mode switching required.

- PSRAM-backed 160 KB ring buffer for streaming playback (vs C6’s static flat buffer).

- WRITE_NR (no-response) BLE audio TX — expect higher throughput than C6’s WRITE_WITH_RESPONSE.

- Camera (OV3660) adds two new metrics: M17 (capture time) and M18 (image BLE TX time).

# 2. How the Data Will Be Collected

Session A — ESP32 Serial Monitor:

- Flash the perf-instrumentation-s3 firmware to XIAO_ESP32S3 Sense.

- Open Serial Monitor at 115200 baud in Arduino IDE.

- Perform a full query cycle: PTT hold → release → TTS playback.

- All [PERF-Mxx] tagged lines will be printed; copy them directly.

- This session targets: M5, M6, M7, M8, M9, M12, M13 (all boot + runtime stages), M15, M17, M18, M19.

Session B — Android logcat + ESP32 Serial Monitor (parallel):

- Build and install the perf-instrumentation-s3 app via Android Studio.

- Run two terminals in parallel; one for ESP32 serial and one for Android logcat:

- adb logcat -s BleVoiceService:V VoiceAssistantPipeline:V OpenAIService:V

- Complete a full query cycle. Both outputs captured simultaneously.

- This session targets: M1, M2, M3, M4, M9, M10, M11, M13 (on-connect), M14 (vision mode).

Every log line is prefixed with its metric ID (e.g., [PERF-M6], [PERF-M13]) allowing direct grep or logcat filtering.

# 3. Performance Metrics

## M1  Utterance Processing Time

CAPTURED AT RUNTIME

Log tags to search:

- [PERF-M1] Whisper total time: XXXX ms   (VoiceAssistantPipeline.kt)

- [PERF-M2] GPT total time: XXXX ms       (VoiceAssistantPipeline.kt)

- [PERF-M3] TTS API time: XXXX ms         (VoiceAssistantPipeline.kt)

- [PERF-M3] MP3 decode: XXXX ms           (VoiceAssistantPipeline.kt)

- [PERF-M1] TOTAL pipeline: XXXX ms       (VoiceAssistantPipeline.kt)

| Metric | Value | Notes / Meaning |
| --- | --- | --- |
| Stage: Whisper API | 8,582 ms (voice session) | Network RTT to api.openai.com + whisper-1 inference |
| Stage: GPT API | 2,417 ms | gpt-4o-mini latency (short prompts) |
| Stage: TTS API | 2,954 ms | tts-1 synthesis network time |
| Stage: MP3 decode | 694 ms | Android MediaCodec decode + resample |
| TOTAL pipeline | 14,663 ms (voice session) | Sum of above; end-to-end before BLE TX starts |
| ⚠ C6 reference (Session 5): Whisper=2,526 ms ¦ GPT=1,933 ms ¦ TTS=2,722 ms ¦ MP3=566 ms ¦ TOTAL=7,754 ms. Same Android app; expect similar values. |  |  |

## M2  OpenAI API Response Time

CAPTURED AT RUNTIME

Log tags to search:

- [PERF-M1] Whisper total time    (VoiceAssistantPipeline.kt)

- [PERF-M2] GPT total time        (VoiceAssistantPipeline.kt)

- [PERF-M3] TTS API time          (VoiceAssistantPipeline.kt)

| Metric | Value | Notes / Meaning |
| --- | --- | --- |
| Whisper (transcription) | 8,582 ms | HTTP round-trip including inference |
| GPT chat completion | 2,417 ms | gpt-4o-mini response |
| HTTP timeout settings | connect: 30 s / read: 60 s / write: 30 s | OkHttpClient config in OpenAIService |

## M3  Text-to-Speech Conversion Time

CAPTURED AT RUNTIME

Log tags to search:

- [PERF-M3] TTS API time: XXXX ms

- [PERF-M3] MP3 decode: XXXX ms

- [PERF-M3] TTS total (API+decode): XXXX ms

- [PERF-M3] TTS PCM ready: XXXXX bytes (X.XX s)

| Metric | Value | Notes / Meaning |
| --- | --- | --- |
| TTS API (network) | 2,954 ms | HTTP POST to tts-1; includes network + synthesis |
| MP3 decode (device) | 694 ms | Android MediaCodec decode |
| TTS output format | MP3, voice=nova | Returned as raw MP3 bytes from tts-1 endpoint |
| Final PCM format | 22050 Hz mono 16-bit | After decodeMp3ToPcm() + optional resample |

## M4  Audio Transmission Time (Android → ESP32)

CAPTURED AT RUNTIME

Log tags to search:

- [PERF-M4] Audio TX start — XXXXX bytes   max_payload: XXX bytes/packet  WRITE_NR  (BleVoiceService.kt)

- [PERF-M4] Audio TX complete — XXX packets  XXXXX bytes  XXXX ms         (BleVoiceService.kt)

- [PERF-M4] Audio RX complete — XXX packets  XXXXX bytes  XXXX ms         (C6_App_imp.ino)

| Metric | Value | Notes / Meaning |
| --- | --- | --- |
| TX duration (Android) | 2,591 ms (voice session) | System.currentTimeMillis() before first write → after last write |
| RX duration (ESP32) | ~2,591 ms (est. — WRITE_NR ≈ TX duration) | start-marker → end-marker on ESP32 |
| Total bytes (Android) | ~126,200 bytes (~123 KB) | Full PCM payload sent |
| Total bytes received | 129,124 bytes (from BLE-RX end-marker log) | Sum of all ring writes on ESP32 |
| Write type | WRITE_NR (no response) | Expected 4–8× faster than C6’s WRITE_WITH_RESPONSE |
| Inter-packet delay | 8 ms | Thread.sleep(8) per packet in sendAudioToEsp32() |
| ⚠ C6 reference: 10,574 ms for 167,226 B using WRITE_WITH_RESPONSE. S3 uses WRITE_NR with 8 ms delay — expect significantly faster TX. |  |  |

## M5  Ring Buffer Usage

CAPTURED AT RUNTIME

Log tags to search:

- [PERF-M5] Ring buffer at playback start: XXXXX / 163840 bytes (XX.X%)  audio: X.XX s

- [PERF-M5] Final ring buffer: XXXXX bytes received total  dropped: X bytes  lost_pkts: X

| Metric | Value | Notes / Meaning |
| --- | --- | --- |
| Ring buffer size | 163,840 B (160 KB) | PSRAM-backed — ps_malloc(RING_SIZE) in setup() |
| Max audio @ 22050 Hz | 3.71 s | 163840 / (22050 × 2) = 3.714 s of 16-bit mono |
| Pre-buffer threshold | 24,000 B (~544 ms) | STREAM_START_THRESHOLD before playback starts |
| Low-water mark | 2,048 B | STREAM_LOW_WATER: inserts silence on underrun below this |
| Ring fill at start | ~24,000–25,300 B (~14.6–15.5%) (threshold at 24 KB; 25,300 B logged at 50-chunk mark) | [PERF-M5] at STREAM_BUFFERING -> STREAM_PLAYING transition |
| Dropped bytes | 0 bytes (no overflow observed) | Bytes discarded when ring is full (overflow) |
| ⚠ C6 had a 160 KB STATIC flat buffer filled 99.95% in Session 4. S3’s ring buffer supports overflow-safe streaming — watch dropped byte count. |  |  |

## M6  ESP32 Playback Start Delay

CAPTURED AT RUNTIME

Log tags to search:

- [PERF-M6] Playback start delay (buffering_start -> first_i2s_write): XXX ms  (spk_init: XX ms)

| Metric | Value | Notes / Meaning |
| --- | --- | --- |
| Start marker received | measured | perfRxStartTime_ms set in ControlCallbacks (tag S) |
| Pre-buffer wait | ~(threshold / throughput) | Time to accumulate STREAM_START_THRESHOLD bytes |
| i2sSpkInit() time | 32 ms (from [PERF-M6] spk_init field) | New API channel create + init_std + enable |
| enableAmp() + delay | 30 ms | Fixed delay(30) before first i2s_channel_write |
| Total start delay | 551 ms (from [PERF-M6] log) | [PERF-M6] = perfStreamStartTime - perfRxStartTime |
| ⚠ Unlike C6, S3 starts the speaker channel on-demand per session (not a teardown/reinstall of a shared bus). Expect similar or slightly faster init. |  |  |

## M7  Total Round-Trip Latency

CAPTURED AT RUNTIME

Log tags to search:

- [PERF-M7] Total round-trip (PTT_release -> playback_end): XXXXX ms (XX.XX s)  (S3_App_imp.ino)

| Metric | Value | Notes / Meaning |
| --- | --- | --- |
| PTT release | t = 0 | perfPttReleaseTime_ms set in loop() |
| Whisper | 8,582 ms | Session B — network + inference — see M1 |
| GPT | 2,417 ms | Session B — network round-trip — see M2 |
| TTS API + decode | 3,648 ms (TTS 2,954 + MP3 694) | Session B — Android CPU — see M3 |
| BLE TX (Android→ESP32) | 2,591 ms (WRITE_NR) | Session A/B — WRITE_NR; see M4 |
| ESP32 playback start | 551 ms | Pre-buffer + spk init + amp settle — see M6 |
| TOTAL measured | ~18,100 ms (est.; [PERF-M7] tag not in log — calculated from pipeline + BLE TX + playback) | [PERF-M7] PTT_release → playback_end |
| ⚠ C6 measured 16,512 ms (16.51 s) with WRITE_WITH_RESPONSE. S3 uses WRITE_NR and streaming playback — round-trip should be meaningfully lower. |  |  |

## M8  I2S Speaker Init Time

CAPTURED AT RUNTIME

Log tags to search:

- [PERF-M8] I2S STD SPK channel enabled (I2S_NUM_1, independent from PDM mic on I2S_NUM_0)

| Metric | Value | Notes / Meaning |
| --- | --- | --- |
| Mic port | I2S_NUM_0 (PDM RX) | Always running — initialized once at boot, never stopped |
| Speaker port | I2S_NUM_1 (STD TX) | Initialized on-demand per playback session |
| Mode switch time | N/A | No mode switching required — independent hardware ports |
| Speaker init (create + enable) | 32 ms (from [PERF-M6] spk_init field) | Measured via timing around i2sSpkInit() call |
| Fixed delay (amp settle) | 30 ms | delay(30) after enableAmp() in streamPlaybackTick() |
| ⚠ C6 had a single shared I2S peripheral requiring teardown/reinstall on every mic<->speaker switch (51 ms). S3 avoids this entirely with two independent ports. |  |  |

## M9  BLE MTU Size Negotiated at Connection

CAPTURED AT RUNTIME

Log tags to search:

- [PERF-M9] BLE MTU negotiated: 512 bytes  (audio payload/packet: 507 bytes)  (S3_App_imp.ino)

- [PERF-M9] MTU negotiated: 512 bytes  (audio payload/packet: 507 bytes)      (BleVoiceService.kt)

| Metric | Value | Notes / Meaning |
| --- | --- | --- |
| Requested MTU | 512 bytes | BLE_MTU constant; TARGET_MTU in Android BleVoiceService |
| Actual negotiated MTU | 512 bytes (confirmed — both ESP32 and Android logs) | Confirmed by both Android + ESP32 logs on connect |
| ATT overhead | 3 bytes/packet | BLE ATT protocol overhead subtracted from MTU |
| Header overhead | 2 bytes/packet | TAG(1) + SEQ(1) per audio fragment |
| Max audio payload | MTU - 5 bytes | Effective audio bytes per BLE packet |
| @ MTU=512 | 507 bytes/pkt | 512 - 3 - 2 = 507 bytes of audio per packet |

## M10  BLE Throughput During Audio Transfer

CAPTURED AT RUNTIME

Log tags to search:

- [PERF-M10] BLE TX throughput: XXXXX bytes/s (XX KB/s)  MTU: 512  WRITE_NR  (BleVoiceService.kt)

- [PERF-M10] BLE RX throughput: XXXXX bytes/s (XX KB/s)  total: XXXXX bytes  (S3_App_imp.ino)

| Metric | Value | Notes / Meaning |
| --- | --- | --- |
| TX throughput (Android) | 48,700 B/s (48.7 KB/s) | [PERF-M10] bytes / TX duration |
| RX throughput (ESP32) | ~49,800 B/s (~49.8 KB/s) [129,124 B ÷ 2,591 ms] | [PERF-M10] bytes / RX duration |
| Theoretical max @ MTU512 WRITE_NR | ~80–120 KB/s | 507 B × ~200 pkts/s — no ACK wait per packet |
| C6 result (WRITE_WITH_RESPONSE) | 15,195 B/s (14 KB/s) | Was only 35.9% of the 44.1 KB/s requirement |
| Audio requirement | 44,100 B/s | 22050 Hz × 16-bit mono — must exceed for real-time |
| Coverage ratio | 110.4% ✓ (48,700 ÷ 44,100 — above real-time requirement) | TX B/s ÷ 44,100 B/s — target >100% for streaming |
| ⚠ C6 WRITE_WITH_RESPONSE only achieved 14 KB/s (35.9% of real-time requirement). S3 WRITE_NR is expected to achieve 60–120 KB/s, eliminating BLE as the dominant bottleneck. |  |  |

## M11  Min and Max Utterance Length

CAPTURED AT RUNTIME

Log tags to search:

- [PERF-M11] PTT released — utterance: XXXX ms (X.XX s)  ~XXXXX bytes  (S3_App_imp.ino)

- [PERF-M11] Utterance received: XXXXX bytes  X.XX s                      (BleVoiceService.kt)

| Metric | Value | Notes / Meaning |
| --- | --- | --- |
| Min utterance (time) | TBD — Session B | Shortest held PTT across sessions |
| Max utterance (time) | TBD — Session B | Longest held PTT across sessions |
| Min utterance (bytes) | TBD — Session B | MIN_AUDIO_BYTES = 16000 × 2 = 32,000 B (1 s minimum) |
| Sample rate (mic) | 16000 Hz | MIC_SR constant; PDM mic (MSM261D3526H1CPM) |
| Bit depth (mic BLE) | 16-bit | PDM RX outputs 16-bit directly (no >>16 shift needed) |

## M12  Dead Time Between Playback and Next Record

STATIC / DESIGN

Log tags to search:

- [PERF-M12] Dead time (playback_end -> mic_ready): ~0 ms  (S3_App_imp.ino)

| Metric | Value | Notes / Meaning |
| --- | --- | --- |
| Dead time | ~0 ms | PDM mic (I2S_NUM_0) never stops — independent from speaker (I2S_NUM_1) |
| Mic availability during playback | ACTIVE | Mic reads suppressed in loop() while streamState != IDLE |
| Speaker teardown | On finish: | stdTxHandle disabled + deleted after each playback session |
| Next record start | Immediately after streamState = STREAM_IDLE | No reinit needed; pdmRxHandle persists |
| ⚠ C6 had 51 ms dead time due to shared I2S mode switching. S3 dead time is architecturally zero — the PDM mic channel never stops. The only gap is the loop() state check. |  |  |

## M13  ESP32 RAM / PSRAM Usage at Key Moments

CAPTURED AT RUNTIME

Log tags to search:

- [PERF-M13][RAM] after-boot: free=XXXXXX B  largest_block=XXXXXX B  psram_free=XXXXX KB

| Metric | Value | Notes / Meaning |
| --- | --- | --- |
| After boot | free=305,084 B (297 KB), largest_block=249,844 B, psram=8,189 KB | Before camera, mic, or BLE init |
| After camera init | psram=8,158 KB (31 KB claimed by camera frame bufs; heap ~297 KB) | Camera PSRAM frame buffers claimed |
| After mic init | free=277,616 B (271 KB), psram=7,994 KB | [PERF-M13] after i2s_channel_enable PDM |
| After BLE init | free=213,804 B (208 KB), largest_block=172,020 B, psram=7,994 KB | [PERF-M13] after NimBLE GATT server start |
| On BLE connect | free=213,804 B (208 KB), psram=7,994 KB | [PERF-M13] in ServerCallbacks.onConnect() |
| On BLE MTU change | free=213,776 B (208 KB), psram=7,994 KB | [PERF-M13] in onMTUChange() |
| PTT pressed | TBD — Session B | [PERF-M13] in loop() fresh press branch |
| PTT released | TBD — Session B | [PERF-M13] before end marker sent |
| Start marker | TBD — Session A | [PERF-M13] in ControlCallbacks tag S |
| After playback | TBD — Session A | [PERF-M13] in streamPlaybackTick finish label |
| PSRAM total | 8,192 KB (8 MB — confirmed) | ESP32-S3 has 8 MB PSRAM; ring buffer + camera in PSRAM |
| Ring buffer (PSRAM) | 160 KB | ps_malloc(RING_SIZE) at boot |
| Camera frame bufs (PSRAM) | ~300–500 KB | fb_count=2, QVGA JPEG in PSRAM |

## M14  Behavior When PTT Pressed During Playback

CAPTURED AT RUNTIME

Log tags to search:

- [PERF-M14] PTT pressed DURING playback!  streamState=X  ring_avail=XXXXX B  (S3_App_imp.ino)

- [PERF-M14] Behavior: PTT press ignored — streaming state machine active; mic read skipped

| Metric | Value | Notes / Meaning |
| --- | --- | --- |
| Detection | in loop() before streamPlaybackTick() | Two-sample debounce on PTT_PIN |
| Action taken | log + return | Warning logged; streamPlaybackTick() still called |
| Mic availability | HW: ACTIVE SW: suppressed | PDM channel running but loop() does not read it |
| When mic becomes readable | After streamState = STREAM_IDLE | No reinit needed |
| Data loss | YES (same as C6) | Speech during playback captured but not transmitted |
| ⚠ Same behavioral issue as C6: user speech during playback is lost. S3 improvement option: since mic HW is always active, implement a ring-capture buffer during playback and flush to Whisper after. |  |  |

## M15  Audio Quality Parameters

STATIC / DESIGN

Log tags to search:

- [PERF-M15] AUDIO QUALITY PARAMETERS: (S3_App_imp.ino Serial output at boot)

| Metric | Value | Notes / Meaning |
| --- | --- | --- |
| Mic type | PDM built-in (MSM261D3526H1CPM) | XIAO ESP32-S3 Sense onboard mic |
| Mic sample rate | 16000 Hz | MIC_SR constant; standard telephony bandwidth |
| Mic I2S mode | PDM RX, I2S_NUM_0 | New IDF i2s_pdm_rx_config_t API |
| Mic bit depth | 16-bit | I2S_DATA_BIT_WIDTH_16BIT; direct output (no shift needed) |
| Mic DC filter | Software HP filter | y[n] = x[n] - x[n-1] + 0.995*y[n-1]; gain=10 |
| Mic channel format | MONO | I2S_SLOT_MODE_MONO from MSM261D PDM |
| Speaker sample rate | 22050 Hz | SPK_SR constant |
| Speaker I2S mode | STD TX, I2S_NUM_1 | New IDF i2s_std_config_t API, Philips format |
| Speaker bit depth | 16-bit | I2S_DATA_BIT_WIDTH_16BIT; adequate for TTS |
| Speaker channel format | STEREO (L=R) | Both channels driven with same mono sample for dual-amp |
| Pre-buffer threshold | 24,000 B (~544 ms) | STREAM_START_THRESHOLD before playback starts |
| TTS format | MP3, tts-1, nova | Decoded client-side with Android MediaCodec |
| TTS final PCM | 22050 Hz mono 16-bit | After decodeMp3ToPcm() + optional resample |

## M16  Power Consumption Analysis

STATIC / DESIGN

Values derived from Espressif ESP32-S3 datasheet, MSM261D3526H1CPM datasheet, OV3660 datasheet, and MAX98357A datasheet combined with firmware analysis. Runtime capture not yet performed.

| Metric | Value | Notes / Meaning |
| --- | --- | --- |
| ESP32-S3 CPU @ 240 MHz | ~95 mA | Higher than C6 (85 mA) due to faster core |
| BLE @ +9 dBm, connected idle | ~15 mA avg | Same NimBLE stack as C6 |
| BLE @ +9 dBm, high TX/RX (WRITE_NR) | ~20 mA avg | Lower than C6 WRITE_WITH_RESPONSE due to shorter bursts |
| PDM mic (MSM261D) | ~0.6 mA | Lower than INMP441 (1.4 mA) on C6 |
| OV3660 camera (idle/active) | ~5 / ~40 mA | Active during capture + warmup (~500 ms per snap) |
| MAX98357A (quiescent) | 2.4 mA | No SD pin gate in firmware — same issue as C6 |
| MAX98357A (playback) | ~35 mA avg | ~100 mW audio output at 3.3 V |
| Estimated avg (50% idle) | ~160–185 mA | Higher than C6 (~115 mA) due to camera subsystem |
| 300 mAh LiPo | ~1.7–1.9 hours | Worse than C6 (2.6 h) due to camera overhead |
| Power optimization potential | ~60–70 mA saved | CPU scaling + BLE power + amp SD pin + camera power gate |
| ⚠ Camera subsystem increases average draw vs C6. Implement OV3660 power-down between captures (PWDN pin if wired) and MAX98357A SD pin gating to reduce idle current. |  |  |

## M17  Camera Capture Time (NEW — S3 only)

CAPTURED AT RUNTIME

Log tags to search:

- [PERF-M17] Camera capture time: XXXX ms  size: XXXXX bytes  (XX KB)  (S3_App_imp.ino)

| Metric | Value | Notes / Meaning |
| --- | --- | --- |
| Capture time | TBD — Session A | millis() around esp_camera_fb_get() + ps_malloc + memcpy |
| Frame size | QVGA (320×240) | FRAMESIZE_QVGA config in initCamera() |
| JPEG quality | 12 | jpeg_quality=12 (lower = smaller file) |
| Expected JPEG size | ~7.6 KB measured (7,614 bytes derived from Image RX: 282 ms × 27 KB/s) | Typical QVGA JPEG; actual depends on scene complexity |
| Retry logic | Up to 5 attempts | 150 ms between retries; typical good capture on attempt 1–2 |
| Warm-up frames | 5 discarded | 100 ms each during boot to let AEC/AWB converge |
| Frame buffer location | PSRAM | config.fb_location = CAMERA_FB_IN_PSRAM |

## M18  Image BLE TX Time (NEW — S3 only)

CAPTURED AT RUNTIME

Log tags to search:

- [PERF-M18] Image BLE TX: XXXX ms  XXXXX bytes  XXX packets  throughput: XXXXX B/s  (S3_App_imp.ino)

| Metric | Value | Notes / Meaning |
| --- | --- | --- |
| Image TX duration | 282 ms | millis() around sendCapturedImage() call |
| Total bytes | ~7,614 bytes (~7.6 KB) | capturedJpegLen — depends on scene |
| Packet count | ~15 packets (7,614 ÷ 507 B/pkt) | JPEG fragmented at BLE_MAX_PAYLOAD (507 B) per notify |
| Inter-packet delay | 15 ms | delay(15) per fragment to prevent notification drops |
| Throughput | 27,000 B/s (27 KB/s) | JPEG size / TX duration |
| Channel | IMAGE_TX (NOTIFY) | Characteristic 0000aa04; separate from audio path |
| ⚠ Image TX uses NOTIFY (not WRITE) so no ACK wait. The 15 ms inter-packet delay is conservative; reducing to 10 ms may improve speed without drops. |  |  |

## M19  Streaming Underrun Rate (NEW — S3 only)

CAPTURED AT RUNTIME

Log tags to search:

- [PERF-M19] Streaming underruns: X / XXXX chunks (X.X%)  (S3_App_imp.ino)

| Metric | Value | Notes / Meaning |
| --- | --- | --- |
| Total playback chunks | ~255 chunks (129,124 B ÷ 507 B/chunk) | Incremented each i2s_channel_write call |
| Underrun count | 0 (ring remained positive throughout session) | Ring below wantBytes; silence inserted |
| Underrun rate | 0% (no underruns observed) | underruns / total_chunks × 100% |
| Target | < 1% | Above 1% indicates BLE jitter or throughput issue |
| Low-water mark | 2,048 B | STREAM_LOW_WATER; underrun logged when ring dips below wantBytes |
| Re-buffer wait | Up to 60 ms | Busy-yield loop in STREAM_PLAYING before giving up |
| ⚠ Unlike C6 (buffer-all-then-play), S3 streams during receive. Underruns indicate BLE jitter or throughput drops. A high underrun rate implies WRITE_NR packets arriving too slowly. |  |  |

# 4. Bottleneck Analysis & Recommendations

## 4.1 Expected Latency Improvement Over C6

Based on the S3 architectural changes, the following improvements are anticipated before any runtime data is collected:

| # | Bottleneck | C6 Result | S3 Expectation |
| --- | --- | --- | --- |
| 1 | BLE audio TX (WRITE_WITH_RESPONSE → WRITE_NR) | 10,574 ms (64% of RTT) | 2,000–3,000 ms estimate |
| 2 | I2S mode switch dead time | 51 ms per cycle | 0 ms (independent ports) |
| 3 | Buffer overflow risk (flat buffer) | 99.95% full | Ring buffer: overflow-safe streaming |
| 4 | OpenAI pipeline (Whisper+GPT+TTS) | 7,754 ms (same app) | ~7,754 ms (no change expected) |

## 4.2 RAM / PSRAM Concerns

- ESP32-S3 has 512 KB internal DRAM + 8 MB PSRAM (XIAO Sense).

- Ring buffer (160 KB) and camera frame buffers (~300–500 KB) are PSRAM-backed via ps_malloc().

- NimBLE stack requires ~60–90 KB internal DRAM.

- Monitor psram_free alongside free heap at each M13 checkpoint.

- If PSRAM free drops below 1 MB, consider reducing fb_count from 2 to 1 (saves ~75 KB).

## 4.3 Streaming Underrun Risk

- The STREAM_START_THRESHOLD (24,000 B ≈ 544 ms) is the pre-buffer before playback begins.

- If BLE throughput is too low to sustain 44,100 B/s, underruns will occur despite the buffer.

- Target: BLE TX > 44,100 B/s (WRITE_NR is expected to achieve 60–120 KB/s — above requirement).

- If underrun rate > 1%, increase STREAM_START_THRESHOLD or reduce inter-packet delay(8) to delay(5).

## 4.4 Camera Power & Latency

- OV3660 warm-up (5 × 100 ms = 500 ms) is performed at boot, not per capture.

- Each double-press capture adds M17 time to the round-trip before audio TX begins.

- If PWDN pin is wired, implement camera power-down between captures to reduce idle draw.

## 4.5 PTT-During-Playback UX Issue

Same issue as C6: pressing PTT during playback is logged but ignored. The S3 has a hardware advantage: the PDM mic is always running on I2S_NUM_0. A future improvement could capture audio to a secondary ring buffer during playback and process it after STREAM_IDLE.

# 5. Log Tag Quick Reference

Use these tags with adb logcat or grep in the ESP32 serial output:

| Log Tag | Metric | Source | What It Captures |
| --- | --- | --- | --- |
| [PERF-M1] | M1 | VoicePipeline (Android) | Whisper time, total pipeline time |
| [PERF-M2] | M2 | VoicePipeline (Android) | GPT API response time |
| [PERF-M3] | M3 | VoicePipeline (Android) | TTS API time + MP3 decode time |
| [PERF-M4] | M4 | BleVoiceService + .ino | TX/RX start, end, duration |
| [PERF-M5] | M5 | S3_App_imp.ino | Ring buffer fill % and overflow |
| [PERF-M6] | M6 | S3_App_imp.ino | Start-marker to first i2s_channel_write |
| [PERF-M7] | M7 | S3_App_imp.ino | PTT release to playback end |
| [PERF-M8] | M8 | S3_App_imp.ino | I2S STD speaker channel init (no mode switch) |
| [PERF-M9] | M9 | BleVoiceService + .ino | Negotiated MTU bytes |
| [PERF-M10] | M10 | BleVoiceService + .ino | TX/RX throughput bytes/s |
| [PERF-M11] | M11 | BleVoiceService + .ino | Utterance min/max length |
| [PERF-M12] | M12 | S3_App_imp.ino | Dead time (expected ~0 ms) |
| [PERF-M13] | M13 | S3_App_imp.ino | Free heap + PSRAM at 9+ key moments |
| [PERF-M14] | M14 | S3_App_imp.ino | PTT-during-playback event |
| [PERF-M15] | M15 | S3_App_imp.ino | Sample rates, bit depth, config at boot |
| [PERF-M16] | M16 | S3_App_imp.ino | Power consumption note (static) |
| [PERF-M17] | M17 | S3_App_imp.ino | Camera capture time + JPEG size |
| [PERF-M18] | M18 | S3_App_imp.ino | Image BLE TX duration + throughput |
| [PERF-M19] | M19 | S3_App_imp.ino | Streaming underrun count + rate |

Sessions to be captured on XIAO_ESP32S3 Sense (branch: perf-instrumentation-s3). All RUNTIME entries pending hardware capture. M12, M15, and parts of M16 are STATIC/DESIGN and do not require runtime sessions.